Chapter 17: Sales and Operations Planning

# Practice Problems

## Multiple Choice

Use the following information to answer Questions 1–21.

Deforrest Marine Motors manufactures engines for the speedboat racing circuit. As part of their annual planning cycle, they forecasted demand for the next four quarters. The number of available days of production and the anticipated demand are given below.

|  | Q1 | Q2 | Q3 | Q4 |
| --- | --- | --- | --- | --- |
| Days of Production | 65 | 68 | 61 | 64 |
| Demand | 1,800 | 2,200 | 1,700 | 2,400 |

They also estimated many of the costs required to conduct operations planning. Some of these key figures are listed below.

| Employees | 30 | Hire Cost | $1,200/employee |
| --- | --- | --- | --- |
| Production Rate | 1 unit/employee/day | Fire Cost | $800/employee |
| Production Cost | $1,000/unit | Subcontracting Cost | $1,800/unit |
| Backorder Cost | $200 | Subcontracting Limit | 400 units maximum |
| Overtime Cost | $1,500 | Inventory Cost | $100/unit/quarter |
| Overtime Limit | <= 25% of Reg. Production | Initial Inventory | 280 units |

1. Given the assumption that Deforrest Marine Motors will maintain the same number of employees (30 employees) for the entire year, what would be the production for the first quarter?

a. 1,520

b. 1,800

c. 1,950

d. 2,100

Ans: C

Difficulty Level: Easy

2. Given the assumption that Deforrest Marine Motors will maintain the same number of employees (30 employees) for the entire year, what would be the production for the second quarter?

a. 1,960

b. 2,040

c. 2,160

d. 2,200

Ans: B

Difficulty Level: Easy

3. Given the assumption that Deforrest Marine Motors will maintain the same number of employees (30 employees) for the entire year, what would be the production for the third quarter?

a. 1,420

b. 1,690

c. 1,740

d. 1,830

Ans: D

Difficulty Level: Easy

4. Given the assumption that Deforrest Marine Motors will maintain the same number of employees (30 employees) for the entire year, what would be the production for the fourth quarter?

a. 1,860

b. 1,920

c. 2,210

d. 2,400

Ans: B

Difficulty Level: Easy

5. Given that the beginning inventory (start of Q1) was 280 units, what would be the average on-hand inventory for the first quarter?

a. 335

b. 355

c. 370

d. 450

Ans: B

Difficulty Level: Medium

6. What would be the average on-hand inventory for the second quarter?

a. 350

b. 360

c. 375

d. 400

Ans: A

Difficulty Level: Medium

7. What would be the average on-hand inventory for the third quarter?

a. 335

b. 355

c. 370

d. 400

Ans: A

Difficulty Level: Medium

8. What would be the average on-hand inventory for the fourth quarter?

a. 160

b. 200

c. 225

d. 255

Ans: A

Difficulty Level: Medium

9. What would be the total on-hand average inventory holding cost for the four quarters?

a. $88,000

b. $92,000

c. $120,000

d. $156,000

Ans: C

Difficulty Level: Hard

10. What was the total cost of producing products across the four quarters?

a. $6,890,000

b. $7,070,000

c. $7,289,000

d. $7,740,000

Ans: D

Difficulty Level: Medium

11. What was the total cost of backordering across the four quarters?

a. $0

b. $9,000

c. $16,000

d. $24,000

Ans: C

Difficulty Level: Medium

12. What was the total cost for this plan?

a. $6,940,000

b. $7,465,000

c. $7,756,000

d. $8,348,000

Ans: C

Difficulty Level: Medium

Use the information below to answer Questions 13–21.

Deforrest Marine Motors manufactures engines for the speedboat racing circuit. As part of their annual planning cycle, they forecasted demand for the next four quarters. The number of available days of production and the anticipated demand are given below.

|  | Q1 | Q2 | Q3 | Q4 |
| --- | --- | --- | --- | --- |
| Days of Production | 65 | 68 | 61 | 64 |
| Demand | 1,800 | 2,200 | 1,700 | 2,400 |

They also estimated many of the costs required to conduct operations planning. Some of these key figures are listed below.

| Employees | 30 | Hire Cost | $1,200/employee |
| --- | --- | --- | --- |
| Production Rate | 1 unit/employee/day | Fire Cost | $800/employee |
| Production Cost | $1,000/unit | Subcontracting Cost | $1,800/unit |
| Backorder Cost | $200 | Subcontracting Limit | 400 units maximum |
| Overtime Cost | $1,500 | Inventory Cost | $100/unit/quarter |
| Overtime Limit | <= 25% of Reg. Production | Initial Inventory | 280 units |

Deforrest Marine Motors wishes to examine a chase strategy—an operations strategy where they hire and fire employees to match demand. They plan on using a policy where if the need were for 21.2 workers, they would hire 22 workers for that quarter.

13. How many workers would Deforrest Marine Motors need for the first quarter?

a. 22

b. 24

c. 26

d. 28

Ans: B

Difficulty Level: Easy

14. How many workers would Deforrest Marine Motors need for the second quarter?

a. 30

b. 31

c. 33

d. 35

Ans: C

Difficulty Level: Easy

15. How many workers would Deforrest Marine Motors need for the third quarter?

a. 28

b. 30

c. 31

d. 34

Ans: A

Difficulty Level: Easy

16. How many workers would Deforrest Marine Motors need for the fourth quarter?

a. 28

b. 33

c. 36

d. 38

Ans: D

Difficulty Level: Easy

17. What would be the total hire and fire cost for this plan across the four quarters?

a. $0

b. $16,400

c. $24,400

d. $29,600

Ans: D

Difficulty Level: Hard

18. What was the total cost of producing products across the four quarters?

a. $7,876,000

b. $8,010,000

c. $8,204,000

d. $8,560,000

Ans: A

Difficulty Level: Medium

19. What was the total cost of backordering across the four quarters?

a. $0

b. $12,600

c. $20,800

d. $28,700

Ans: A

Difficulty Level: Medium

20. What would be the total on-hand average inventory holding cost for the four quarters?

a. $33,200

b. $36,200

c. $37,600

d. $40,000

Ans: D

Difficulty Level: Medium

21. What was the total cost for this plan?

a. $7,762,400

b. $7,945,600

c. $8,989,000

d. $9,145,000

Ans: B

Difficulty Level: Hard

Use the information below to answer Questions 22–38.

Deforrest Marine Motors manufactures engines for the speedboat racing circuit. As part of their annual planning cycle, they forecasted demand for the next four quarters. The number of available days of production and the anticipated demand are given below.

|  | Q1 | Q2 | Q3 | Q4 |
| --- | --- | --- | --- | --- |
| Days of Production | 65 | 68 | 61 | 64 |
| Demand | 1,800 | 2,200 | 1,700 | 2,400 |

They also estimated many of the costs required to conduct operations planning. Some of these key figures are listed below.

| Employees | 30 | Hire Cost | $1,200/employee |
| --- | --- | --- | --- |
| Production Rate | 1 unit/employee/day | Fire Cost | $800/employee |
| Production Cost | $1,000/unit | Subcontracting Cost | $1,800/unit |
| Backorder Cost | $200 | Subcontracting Limit | 400 units maximum |
| Overtime Cost | $1,500 | Inventory Cost | $100/unit/quarter |
| Overtime Limit | <= 25% of Reg. Production | Initial Inventory | 280 units |

Deforrest Marine Motors wishes to examine a mixed strategy. They reduced their workforce to 25 employees. If the regular production and the inventory is unable to meet demand, they will employ (in that quarter) the maximum amount of overtime. Should that be insufficient, the firm will use subcontracting up to the point of exactly meeting demand.

22. What would be the regular-time production level for the first quarter?

a. 1,575

b. 1,625

c. 1,675

d. 1,725

Ans: B

Difficulty Level: Easy

23. What would be the regular-time production level for the second quarter?

a. 1,700

b. 1,860

c. 1,960

d. 2,200

Ans: A

Difficulty Level: Easy

24. What would be the regular-time production level for the third quarter?

a. 1,430

b. 1,500

c. 1,525

d. 1,600

Ans: C

Difficulty Level: Easy

25. What would be the regular-time production level for the fourth quarter?

a. 1,430

b. 1,500

c. 1,525

d. 1,600

Ans: D

Difficulty Level: Easy

26. What would be the required overtime production, if any, in the first quarter?

a. 0

b. 145

c. 175

d. 260

Ans: A

Difficulty Level: Medium

27. What would be the required overtime production, if any, in the second quarter?

a. 350

b. 380

c. 425

d. 440

Ans: C

Difficulty Level: Medium

28. What would be the required overtime production, if any, in the third quarter?

a. 290

b. 381

c. 420

d. 440

Ans: B

Difficulty Level: Medium

29. What would be the required overtime production, if any, in the fourth quarter?

a. 175

b. 225

c. 325

d. 400

Ans: D

Difficulty Level: Medium

30. Given that the beginning inventory (start of Q1) was 280 units, what would be the average on-hand inventory (round up if necessary) for the first quarter?

a. 0

b. 143

c. 280

d. 300

Ans: B

Difficulty Level: Medium

31. What would be the average on-hand inventory (round up if necessary) for the second quarter?

a. 68

b. 90

c. 120

d. 143

Ans: A

Difficulty Level: Medium

32. What would be the average on-hand inventory (round up if necessary) for the third quarter?

a. 55

b. 88

c. 134

d. 202

Ans: C

Difficulty Level: Medium

33. What would be the average on-hand inventory (round up if necessary) for the fourth quarter ?

a. 0

b. 118

c. 165

d. 183

Ans: B

Difficulty Level: Medium

34. What would be the total cost for regular-time production across the four quarters?

a. $3,230,000

b. $4,400,000

c. $6,450,000

d. $7,990,000

Ans: C

Difficulty Level: Easy

35. What would be the total cost for overtime production across the four quarters?

a. $235,600

b. $398,100

c. $712,300

d. $1,809,375

Ans: D

Difficulty Level: Hard

36. What would be the total on-hand average inventory holding cost for the four quarters?

a. $51,300

b. $178,100

c. $234,800

d. $401,600

Ans: A

Difficulty Level: Medium

37. What would be the total subcontracting cost, if any, for the four quarters?

a. $102,800

b. $182,000

c. $295,200

d. $456,000

Ans: C

Difficulty Level: Hard

38. What was the total cost for this plan?

a. $8,302,500

b. $8,605,875

c. $8,903.400

d. $9,102,700

Ans: B

Difficulty Level: Medium

Use the following information to answer Questions 39–50.

Xebex Technologies Corporation (XTC) manufactures a number of navigation packages for pleasure boats. Their plant in Alabama wants to try a fixed production schedule for the next 6 months. When regular-time production and inventory cannot meet demand, they plan on first using overtime, and if that is insufficient, they will subcontract out in order to meet demand. The forecast for the next 6 months and key production data are provided below.

|  | Jan | Feb | Mar | Apr | May | Jun |
| --- | --- | --- | --- | --- | --- | --- |
| Demand | 700 | 600 | 400 | 650 | 850 | 1,000 |

| Production Rate | 650 units/month | Fire Cost | $800/employee |
| --- | --- | --- | --- |
| Production Cost | $100/units | Subcontracting Cost | $200/unit |
| Backorder Cost | $175/unit | Subcontracting Limit | 200 units maximum |
| Overtime Cost | $150/unit | Average Inventory Cost | $20/unit/month |
| Overtime Limit | 100 units | Initial Inventory | 100 units |

39. Given the fixed production rate of 650 units per month and an initial inventory level of 100 units, what would be the average on-hand inventory for January?

a. 55

b. 75

c. 100

d. 150

Ans: B

Difficulty Level: Easy

40. What would be the average on-hand inventory for March?

a. 75

b. 150

c. 175

d. 225

Ans: D

Difficulty Level: Easy

41. What would be the total inventory holding cost for the 6 months?

a. $0

b. $21,000

c. $105,000

d. $205,000

Ans: B

Difficulty Level: Medium

42. What would be the regular-time production cost for the 6 months?

a. $120,000

b. $390,000

c. $650,000

d. $780,000

Ans: B

Difficulty Level: Easy

43. What months, if any, did XTC require the use of overtime production?

a. January

b. March, April, and June

c. May and June

d. June

Ans: D

Difficulty Level: Medium

44. What months, if any, did XTC require the use of subcontracting production?

a. January

b. March, April, and June

c. May and June

d. June

Ans: D

Difficulty Level: Medium

45. What would be the overtime time production cost, if any, for the 6 months?

a. $0

b. $10,000

c. $15,000

d. $25,000

Ans: C

Difficulty Level: Medium

46. What would be the subcontracting cost, if any, for the 6 months?

a. $0

b. $10,000

c. $15,000

d. $20,000

Ans: D

Difficulty Level: Medium

47. As an alternative to a fixed production schedule, Xebex Technologies Corporation (XTC) considers an operations plan where they fluctuate the number of employees. Presently, XTC has 25 employees, and each employee can produce 25 units per month. The cost of laying off an employee is $800 per employee, and the cost of hiring an employee is $1,000 per employee.

What would be the hiring cost for this program?

a. $0

b. $15,000

c. $20,000

d. $24,000

Ans: D

Difficulty Level: Medium

48. What would be the total layoff cost for this program?

a. $0

b. $1,800

c. $7,200

d. $9,600

Ans: C

Difficulty Level: Medium

49. What would be the total cost of subcontracting?

a. $0

b. $1,200

c. $3,000

d. $3,600

Ans: A

Difficulty Level: Easy

50. What would be the total cost of this program (fluctuating personnel)?

a. $402,500

b. $442,200

c. $601,800

d. $728,000

Ans: B

Difficulty Level: Hard