Chapter 18S: Capacity Requirements Planning, MRP II, ERP, and DRP

# Practice Problems

## Multiple Choice

Use the following information to answer Questions 1–8.

Zontar Fabrications is planning the next day’s stamping operations. They have six jobs lined up for tomorrow. The volume of required production, the setup times and the stamping (run) times for the six products are listed below.

|  | Order | Setup | Runtime |
| --- | --- | --- | --- |
| Job | Size | (Min.) | (Min.) |
| A-0 | 600 | 6 | 0.25 |
| A-2 | 1,000 | 10 | 0.20 |
| B-3 | 1,200 | 15 | 0.20 |
| B-5 | 800 | 9 | 0.30 |
| C-2 | 1,400 | 12 | 0.25 |
| C-6 | 900 | 8 | 0.20 |

1. What would be the total required time to produce Job A-0?

a. 156

b. 177

c. 192

d. 210

Ans: A

Difficulty Level: Easy

2. What would be the total required time to produce Job A-2?

a. 192

b. 210

c. 255

d. 281

Ans: B

Difficulty Level: Easy

3. What would be the total required time to produce Job B-3?

a. 233

b. 249

c. 255

d. 277

Ans: C

Difficulty Level: Easy

4. What would be the total required time to produce Job B-5?

a. 233

b. 241

c. 249

d. 255

Ans: C

Difficulty Level: Easy

5. What would be the total required time to produce Job C-2?

a. 255

b. 279

c. 350

d. 362

Ans: D

Difficulty Level: Easy

6. What would be the total required time to produce Job C-6?

a. 177

b. 188

c. 192

d. 210

Ans: B

Difficulty Level: Easy

7. Zontar Fabrications has two machines in its stamping operations, each of which functions at an 87% efficiency. Zontar runs two 8-hour shifts. What would be the stamping operations idle capacity, if any?

a. 0 minutes

b. 23 minutes

c. 162 minutes

d. 250 minutes

Ans: D

Difficulty Level: Medium

8. What would be the utilization of the stamping operation for these six jobs?

a. 0%

b. 15%

c. 60%

d. 85%

Ans: D

Difficulty Level: Medium

Use the following information to answer Questions 9–12.

Glencoe Industries will be manufacturing four products for the frames of military motorcycles next month. They are operating on an 8-hour-day shift and will be open 23 days next month. Data for their monthly demand and production standards are provided below.

|  |  |  |  | Standard Labor Hours per Unit per Employee |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Product | Demand | Materials |  | Fabrication | Welding | Assembly |
| A | 200 | 6.9 lb./unit |  | 0.9 | 0.3 | 1.1 |
| B | 550 | 5.0 lb./unit |  | 1.4 | 0.2 | 0.8 |
| C | 375 | 9.2 lb./unit |  | 2.0 | 0.4 | 1.3 |
| D | 425 | 2.9 lb./unit |  | 1.6 | 0.2 | 0.8 |

9. If Glencoe purchased 9,000 pounds of materials what would be the surplus or deficit in materials?

a. deficit of 187.5 pounds

b. deficit of 98.5 pounds

c. neither deficit nor surplus

d. surplus of 187.5 pounds

Ans: D

Difficulty Level: Medium

10. Given the demand for all four products, what would be the number of workers required in the fabrication area? (Round up.)

a. 8

b. 10

c. 13

d. 19

Ans: C

Difficulty Level: Hard

11. Given the demand for all four products, what would be the number of workers required in the welding area? (Round up.)

a. 3

b. 6

c. 9

d. 13

Ans: A

Difficulty Level: Hard

12. Given the demand for all four products, what would be the number of workers required in the assembly area? (Round up.)

a. 7

b. 9

c. 13

d. 17

Ans: B

Difficulty Level: Hard

Use the following information to answer Questions 13–23.

YoYo Dyne produces several types of small quadrocoptor drones for commercial and military use. They are planning for the next 6 months’ production. The forecasted demand for their best three selling models is given below.

| Product | January | February | March | April | May | June |
| --- | --- | --- | --- | --- | --- | --- |
| Sarissa | 300 | 400 | 450 | 600 | 550 | 500 |
| Javelin | 125 | 300 | 180 | 290 | 400 | 600 |
| Pilum | 100 | 120 | 135 | 175 | 200 | 210 |

In order to plan for production, YoYo Dyne gathered information of standard labor times for the five major operations required. These data are given below.

|  | Standard Hours per Unit per Employee |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Product | Preparation | Burring | Fabrication | Assembly | Inspection |
| Sarissa | 0.6 | 0.4 | 0.9 | 1.9 | 0.2 |
| Javelin | 0.8 | 0.2 | 1.2 | 2.2 | 0.2 |
| Pilum | 0.9 | 0.3 | 1.4 | 2.5 | 0.3 |

13. How many hours would be required to meet January’s demand for Sarissa drones in Preparation?

a. 160

b. 180

c. 190

d. 210

Ans: B

Difficulty Level: Easy

14. How many hours would be required to meet February’s demand for Javelin drones in Burring?

a. 60

b. 76

c. 84

d. 96

Ans: A

Difficulty Level: Easy

15. How many hours would be required to meet March’s demand for Pilum drones in Fabrication?

a. 98

b. 156

c. 189

d. 226

Ans: C

Difficulty Level: Easy

16. How many hours would be required to meet April’s demand for Sarissa drones in Assembly?

a. 920

b. 960

c. 1050

d. 1140

Ans: D

Difficulty Level: Easy

17. How many hours would be required to meet May’s demand for Javelin drones in Inspection?

a. 80

b. 94

c. 110

d. 128

Ans: A

Difficulty Level: Easy

18. What would be the total number of hours required in Preparation during January for the three types of drones?

a. 165

b. 370

c. 540

d. 740

Ans: B

Difficulty Level: Medium

19. What would be the total number of hours required in Burring during March for the three types of drones?

a. 167.5

b. 196.0

c. 233.0

d. 256.5

Ans: D

Difficulty Level: Medium

20. What would be the total number of hours required in Inspection during June for the three types of drones?

a. 188

b. 283

c. 312

d. 425

Ans: B

Difficulty Level: Medium

21. Across the 6 months, what would be the total required number of hours to produce all of the Sarissa drones?

a. 8.720

b. 9.620

c. 10,340

d. 11,200

Ans: D

Difficulty Level: Hard

22. Across the 6 months, what would be the total required number of hours to produce all of the Javelin drones?

a. 7,120

b. 7,890

c. 8,717

d. 9.230

Ans: C

Difficulty Level: Hard

23. Across the 6 months, what would be the total required number of hours to produce all of the Pilum drones?

a. 4,340

b. 5,076

c. 5,970

d. 6,380

Ans: B

Difficulty Level: Hard

Use the following additional information to answer Questions 24–30.

YoYo Dyne produces several types of small quadrocoptor drones for commercial and military use. They are planning for the next 6 months’ production. The forecasted demand for their best three selling models is given below.

| Product | January | February | March | April | May | June |
| --- | --- | --- | --- | --- | --- | --- |
| Sarissa | 300 | 400 | 450 | 600 | 550 | 500 |
| Javelin | 125 | 300 | 180 | 290 | 400 | 600 |
| Pilum | 100 | 120 | 135 | 175 | 200 | 210 |

In order to plan for production, YoYo Dyne gathered information of standard labor times for the five major operations required. These data are given below.

|  | Standard Hours per Unit per Employee |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Product | Preparation | Burring | Fabrication | Assembly | Inspection |
| Sarissa | 0.6 | 0.4 | 0.9 | 1.9 | 0.2 |
| Javelin | 0.8 | 0.2 | 1.2 | 2.2 | 0.2 |
| Pilum | 0.9 | 0.3 | 1.4 | 2.5 | 0.3 |

YoYo Dyne prides itself on operating in a lean fashion. This is particularly true with respect to personnel. YoYo Dyne plans on running two 8-hour shifts every day and to operate 25 days every month. They are planning to assign 1 worker in each shift for Preparation; 1 worker in each shift for Burring; 3 workers in each shift for Fabrication; 5 workers in each shift for Assembly; and 1 worker in each shift for Inspection.

24. Given the 1 worker in each shift for Preparation, what would be the utilization in Preparation in January?

a. 80%

b. 93%

c. 100%

d. 120%

Ans: B

Difficulty Level: Medium

25. Assuming that a worker performs an 8-hour day for 25 days a month, what would be the total number of required workers across the 2 shifts in Preparation for June? (If needed, round up.)

a. 1

b. 3

c. 5

d. 6

Ans: C

Difficulty Level: Medium

26. The plan is to have 1 worker in each shift for Burring operations. In how many of the 6 months would that be sufficient?

a. 0

b. 2

c. 4

d. 6

Ans: D

Difficulty Level: Medium

27. Given the 1 worker in each shift for Inspection, what would be the utilization in January?

a. 29%

b. 45%

c. 75%

d. 93%

Ans: A

Difficulty Level: Hard

28. Given the 5 workers in each shift for Assembly, what would be the utilization in June?

a. 40%

b. 80%

c. 100%

d. 140%

Ans: D

Difficulty Level: Hard

29. How many workers in each shift would be required in Assembly to meet the expected demand in June?

a. 5

b. 6

c. 7

d. 8

Ans: C

Difficulty Level: Hard

30. The plan is to have 1 worker in each shift for Inspection operations. In how many of the 6 months would that be sufficient?

a. 0

b. 2

c. 4

d. 6

Ans: D

Difficulty Level: Medium

Use the following information to answer Questions 31–50.

Precision Pumps’ plant in Dayton, Ohio, produces three types of commercial pumps. Manufacturing of the pumps is broken down into three broad categories: Preparation, Assembly, and Inspection. The per unit standard machine and labor requirements for the three types of pumps in each category is provided below.

|  | Preparation |  | Assembly |  | Inspection |  |
| --- | --- | --- | --- | --- | --- | --- |
| Product | Machine | Labor | Machine | Labor | Machine | Labor |
| K-110 | 4 | 2 | 6 | 12 | 1 | 3 |
| K-220 | 3 | 5 | 8 | 15 | 2 | 4 |
| M-440 | 10 | 9 | 10 | 8 | 4 | 6 |

The plant manager is looking at the production schedule for the next week. The daily demands for the three types of pumps are as follows.

| Product | Monday | Tuesday | Wednesday | Thursday | Friday |
| --- | --- | --- | --- | --- | --- |
| K-110 | 25 | 35 | 40 | 51 | 70 |
| K-220 | 35 | 32 | 40 | 27 | 25 |
| M-440 | 19 | 12 | 15 | 18 | 15 |

31. How many hours of Machine time in Preparation would be required to produce Monday’s demand of K-110?

a. 4

b. 25

c. 100

d. 200

Ans: C

Difficulty Level: Easy

32. How many hours of Labor time in Assembly would be required to produce Tuesday’s demand of K-220?

a. 32

b. 180

c. 360

d. 480

Ans: D

Difficulty Level: Easy

33. How many hours of Machine time in Inspection would be required to produce Wednesday’s demand of M-440?

a. 25

b. 60

c. 100

d. 125

Ans: B

Difficulty Level: Easy

34. How many hours of Labor time in Preparation would be required to produce Thursday’s demand of K-110?

a. 60

b. 92

c. 102

d. 122

Ans: C

Difficulty Level: Easy

35. How many hours of Machine time in Assembly would be required to produce Friday’s demand of K-220?

a. 180

b. 200

c. 280

d. 360

Ans: B

Difficulty Level: Easy

36. How many hours of Labor time in Preparation would be required to produce Monday’s demand of M-440?

a. 86

b. 94

c. 100

d. 114

Ans: D

Difficulty Level: Easy

37. What would be the total number of Machine hours required in Preparation to produce all three products on Monday?

a. 350

b. 395

c. 445

d. 495

Ans: B

Difficulty Level: Hard

38. What would be the total number of Labor hours required in Assembly to produce all three products on Tuesday?

a. 676

b. 724

c. 882

d. 996

Ans: D

Difficulty Level: Hard

39. What would be the total number of Machine hours required in Inspection to produce all three products on Wednesday?

a. 120

b. 180

c. 240

d. 270

Ans: B

Difficulty Level: Hard

40. What would be the total number of Labor hours required in Preparation to produce all three products on Thursday?

a. 277

b. 303

c. 399

d. 421

Ans: C

Difficulty Level: Hard

41. The operations manager at Precision Pumps has scheduled that there will be 500 Machine hours and 600 Labor hours available for Preparation, 800 Machine hours and 1,200 Labor hours available for Assembly, and 200 Machine hours and 400 Labor hours available for Inspection. What will be the Machine utilization across all three products in Preparation for Monday?

a. 61.3%

b. 67.8%

c. 79.0%

d. 88.2%

Ans: C

Difficulty Level: Medium

42. The operations manager at Precision Pumps has scheduled that there will be 500 machine hours and 600 Labor hours available for Preparation, 800 Machine hours and 1,200 Labor hours available for Assembly, and 200 Machine hours and 400 labor hours available for Inspection. What will be the Labor utilization across all three products in Preparation for Wednesday?

a. 58.7%

b. 69.2%

c. 77.6%

d. 82.8%

Ans: B

Difficulty Level: Medium

43. The operations manager at Precision Pumps has scheduled that there will be 500 Machine hours and 600 Labor hours available for Preparation, 800 Machine hours and 1,200 Labor hours available for Assembly, and 200 Machine hours and 400 Labor hours available for Inspection. What will be the Machine utilization across all three products in Preparation for Friday?

a. 90.7%

b. 93.4%

c. 97.8%

d. 101.0%

Ans: D

Difficulty Level: Medium

44. The operations manager at Precision Pumps has scheduled that there will be 500 Machine hours and 600 Labor hours available for Preparation, 800 Machine hours and 1,200 Labor hours available for Assembly, and 200 Machine hours and 400 Labor hours available for Inspection. What will be the Labor utilization across all three products in Assembly for Monday?

a. 33.0%

b. 45.6%

c. 67.2%

d. 88.8%

Ans: A

Difficulty Level: Medium

45. The operations manager at Precision Pumps has scheduled that there would 500 Machine hours and 600 Labor hours available for Preparation, 800 Machine hours and 1,200 Labor hours available for Assembly, and 200 Machine hours and 400 Labor hours available for Inspection. What will be the Machine utilization across all three products in Assembly for Wednesday?

a. 33.0%

b. 45.6%

c. 67.2

d. 88.8%

Ans: D

Difficulty Level: Medium

46. The operations manager at Precision Pumps has scheduled that there will be 500 Machine hours and 600 Labor hours available for Preparation; 800 Machine hours and 1,200 Labor hours available for Assembly, and 200 Machine hours and 400 Labor hours available for Inspection. What will be the Labor utilization across all three products in Assembly for Friday?

a. 78.2

b. 92.8%

c. 100.0%

d. 105.3%

Ans: D

Difficulty Level: Medium

47. The operations manager at Precision Pumps has scheduled that there will be 500 Machine hours and 600 Labor hours available for Preparation, 800 Machine hours and 1,200 Labor hours available for Assembly, and 200 Machine hours and 400 Labor hours available for Inspection. What will be the Machine utilization across all three products in Inspection for Monday?

a. 77.2%

b. 80.3%

c. 85.5%

d. 99.0%

Ans: C

Difficulty Level: Medium

48. The operations manager at Precision Pumps has scheduled that there will be 500 Machine hours and 600 Labor hours available for Preparation, 800 Machine hours and 1,200 Labor hours available for Assembly, and 200 Machine hours and 400 Labor hours available for Inspection. What will be the Labor utilization across all three products in Preparation for Wednesday?

a. 78.6%

b. 81.7%

c. 88.3%

d. 92.5%

Ans: D

Difficulty Level: Medium

49. The operations manager at Precision Pumps has scheduled that there will be 500 Machine hours and 600 Labor hours available for Preparation, 800 Machine hours and 1,200 Labor hours available for Assembly, and 200 Machine hours and 400 Labor hours available for Inspection. What would be the Machine utilization across all three products in Preparation for Monday?

a. 87.5%

b. 90.0%

c. 96.2%

d. 98.3%

Ans: B

Difficulty Level: Medium

50. What would be the average Machine utilization for all three products in Preparation across the five days?

a. 78.5%

b. 82.3%

c. 86.0%

d. 93.4%

Ans: C

Difficulty Level: Hard